sac_v5: weights 50-20-2, fixed building and heat pump models, parabolic reward around 21 deg.

sac_v6: normalized obs space, building temp limit detection implemented - too many cycles, need to punish cycling more

sac_v7: 50-20-25, more weight put on cycling

sac_v8: solar and internal gains added with time features in obs space

sac_v9: scaling decreased to 1/100 and the heating cutoff changed to HP power rather than ambient temperature

sac_v10: gains added to obs space

sac_v11: gains recalculated with greater shading

sac_v13:

|  | comfort % | range C | mean C | median C | std dev C | mean abs dev K | too cold hours h | too hot hours h | electricity kWh | thermal energy kWh | cost euro | per energy ct/kWh | cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2024-25 | 83.2% | 19.4-29.4 | 21.58 | 21.08 | 1.45 | 0.72 | 16 (0.3%) | 831 (16.5%) | 5054.1 | 24710.9 | 1777.96 | 35.18 | 719 |
| 2025-26 | 85.3% | 19.1-27.8 | 21.45 | 21.04 | 1.27 | 0.62 | 19 (0.4%) | 721 (14.3%) | 5439.5 | 25111.0 | 1832.71 | 33.69 | 742 |